AgriBot Vision System

Testing & Hardware Guide

Software Testing Procedures + Hardware Integration Roadmap

PART 1 — SOFTWARE TESTING GUIDE

# 1. Prerequisites

Before running any tests, ensure the following Python packages are installed:

| Package | Min Version | Install Command |
| --- | --- | --- |
| Python | 3.10+ | python.org |
| pytest | 9.0+ | pip install pytest |
| opencv-contrib-python | 4.10+ | pip install opencv-contrib-python |
| pydantic | 2.0+ | pip install pydantic |
| flask | 3.0+ | pip install flask |
| numpy | 2.0+ | pip install numpy |
| pyzbar | any | pip install pyzbar |
| ultralytics (YOLO) | 8.0+ | pip install ultralytics |
| python-can | 4.0+ | pip install python-can |

Install all at once:

pip install pytest opencv-contrib-python pydantic flask numpy pyzbar ultralytics python-can

Working directory for all commands:

E:\TTS\Projects\Web-Applications\Image-processing

# 2. Unit Tests (Automated)

The full test suite covers 11 modules across 66 test cases. Run with:

python -m pytest tests/ -v

Expected output: 66 passed

## 2.1  Test Modules

| Test File | What It Tests | Cases |
| --- | --- | --- |
| test_aruco_detector.py | ArUco detector init and empty frame handling | 2 |
| test_barcode_detector.py | Barcode detector init and empty frame handling | 2 |
| test_command_queue.py | Priority ordering, push/pop, queue full, clear, timestamps | 7 |
| test_config.py | Config manager defaults and invalid path handling | 2 |
| test_config_schema.py | Pydantic schema — ranges, CAN data, normalization | 11 |
| test_dashboard_api.py | Flask /status, /logs, /metrics/history, /metrics routes | 12 |
| test_decision_engine.py | Obstacle blocking, turn trigger, steering, recovery, E-STOP | 9 |
| test_frame_pipeline.py | Empty frame guard, output shape, stability score range | 7 |
| test_navigation_filter.py | EMA smoothing, dead-zone, reset, hot-reload | 6 |
| test_safety.py | Stale-frame E-STOP, auto-recovery, reset-on-resume | 3 |
| test_vehicle_state.py | State machine init, cooldown, emergency override | 3 |

## 2.2  Useful Test Flags

Run a single module:

python -m pytest tests/test_decision_engine.py -v

Run only tests matching a keyword:

python -m pytest tests/ -k "obstacle" -v

Stop on first failure:

python -m pytest tests/ -x

Show coverage (requires pytest-cov):

pip install pytest-cov

python -m pytest tests/ --cov=. --cov-report=term-missing

# 3. Code Quality Checks

## 3.1  Linting (flake8)

python -m flake8 .

Expected: no output (zero warnings). Configuration is in setup.cfg — max-line-length=88, E203/W503/E501 suppressed for black compatibility.

## 3.2  Type Checking (mypy)

python -m mypy . --ignore-missing-imports

Key files verified clean:

- config/schema.py — pydantic models with plugin

- navigation/navigation_filter.py — float return types

- controllers/command_queue.py — Optional[float] timeout

- detectors/aruco_detector.py — Optional[int] dict_type

- detectors/yolo_detector.py — None guard on res.boxes

- detectors/feature_detector.py — OpenCV stub suppressions

- navigation/command_visualizer.py — ty narrowing fix

- vision/pose_stability_analyzer.py — float cast on numpy return

## 3.3  Formatting (black)

pip install black

python -m black . --check

To auto-format:

python -m black .

# 4. Simulation Mode Testing  (No Camera Required)

Simulation mode generates synthetic ArUco markers internally. Use this to test the full navigation pipeline without any hardware.

python main.py --simulate

What to verify:

- Window titled "Robotics Vision Output" opens

- STATE label shows DRIVING

- Synthetic ArUco markers appear as the robot approaches

- STATE transitions DRIVING → TURNING when a marker is close

- Web dashboard at http://localhost:5000 shows live status

- Log panel in dashboard updates in real time

- FPS chart draws data points every 0.5 s

## 4.1  Simulation with Stress Test

Tests frame drops and added lag to simulate poor network or hardware conditions:

python main.py --simulate --stress

Expected: occasional "Frame dropped" log entries, FPS may drop, system should remain stable.

# 5. Replay Mode Testing

Replay mode feeds a previously recorded session back through the pipeline — useful for debugging specific scenarios without re-running hardware.

Prerequisites: a recording directory must exist under recordings/.

Run at normal speed:

python main.py --replay recordings/<session_dir>

Run at 2x speed (fast-forward):

python main.py --replay recordings/<session_dir> --speed 2.0

Run at 0.5x speed (slow motion):

python main.py --replay recordings/<session_dir> --speed 0.5

What to verify:

- Frames play back in sequence

- Detection results match what was originally recorded

- Log shows "Replay finished." when the last frame is reached

- Speed flag visibly changes playback rate

# 6. Dashboard Testing

Start the system in simulation mode, then open a browser to the following endpoints:

| URL | Description |
| --- | --- |
| http://localhost:5000 | Main UI — live video, status, logs, chart |
| http://localhost:5000/status | JSON status snapshot |
| http://localhost:5000/logs | Last 200 log entries (JSON) |
| http://localhost:5000/metrics/history | FPS/latency ring buffer (JSON) |
| http://localhost:5000/metrics | Prometheus scrape endpoint |

### Dashboard UI Checklist

- Live video feed renders (MJPEG stream)

- Vehicle state chip updates colour — green=DRIVING, yellow=TURNING, red=STOPPED

- FPS and latency values update every 0.5 s

- Uptime counter increments

- Connection dot is green when running, red when server unreachable

- Log panel scrolls to latest entry automatically

- FPS / Latency chart draws lines as data accumulates

- No obstacles — "None detected" shown in obstacle section

Prometheus metrics test:

curl http://localhost:5000/metrics

Expected plain-text response includes:

agribot_fps 28.5

agribot_estop_total 0

agribot_yolo_faulted 0

# 7. Config Hot-Reload Testing

- Start the system:  python main.py --simulate

- Open config/default.yaml in any text editor

- Change navigation.smoothing_alpha from 0.3 to 0.8 and save

- Within 2 seconds, observe in the terminal:
   Config file changed — reloading.
   Live components updated from reloaded config.

- Change it back and verify the reload fires again

On Linux/macOS you can also trigger a manual reload with:

kill -HUP <pid>

# 8. Robot ID Testing

Test that the CLI flag takes precedence over the config file value:

python main.py --simulate --robot-id robot-42

Verify:

- Every log line starts with: robot-42 |

- http://localhost:5000/status returns: "robot_id": "robot-42"

- Prometheus endpoint shows agribot labels stamped with robot-42

# 9. Frame-Skip Testing

Simulate a slow vision pipeline with a minimal queue depth:

python main.py --simulate --max-queue-depth 1

Watch for log lines like:

Frame queue full — dropping stale frame (processing lag).

This confirms the frame-skip strategy is active and the main loop never blocks.

# 10. Safety Monitor Testing

## 10.1  Auto-Recovery ON (default)

The system keeps running when a stale-frame E-STOP fires and recovers automatically when frames resume.

python -m pytest tests/test_safety.py::test_safety_monitor_auto_recovery_resets_on_frame -v

## 10.2  Auto-Recovery OFF (fail-safe)

Set in config: safety.enable_auto_recovery: false, then run:

python -m pytest tests/test_safety.py::test_safety_monitor_stale_frame_no_auto_recovery -v

Confirm SafetyViolationError is raised — this is the expected fail-safe behaviour for unattended deployments.

# 11. YOLO Obstacle Detection Testing

YOLO requires ultralytics (model is auto-downloaded on first run). You need a real USB camera for this test — YOLO detects real-world objects in the live feed.

python main.py

Note: Use python main.py (live camera), not --simulate. Simulation generates synthetic ArUco markers only and does not produce real obstacle detections.

Point your camera at a person. Verify:

- Bounding box appears around the detected object

- Label and confidence % shown above the box

- If bbox area > 12% of frame: box turns red, "STOP" text appears below

- Dashboard obstacle list shows the detection with CRITICAL badge

- Vehicle state transitions to STOPPED

### Graceful Degradation Test

1. In config/default.yaml set:  detectors.yolo.model: "nonexistent.pt"

2. Restart — verify "Failed to load YOLO model" in logs, system continues

3. Dashboard shows the yellow YOLO fault warning banner

# Prometheus should show:

agribot_yolo_faulted 1

# 12. Recording and Replay Workflow

Enable recording in config/default.yaml:

recording:

enabled: true

output_dir: "recordings"

Run a simulation session, then press Q to quit.

python main.py --simulate

Verify the recordings/ directory contains:

recordings/<timestamp>/

raw/            *.jpg frames

annotated/      *.jpg frames with overlays

session.json    metadata per frame

test_report.json

Replay and confirm behaviour matches the original:

python main.py --replay recordings/<timestamp>

PART 2 — HARDWARE INTEGRATION ROADMAP

# Phase 1 — Hardware Procurement

| Component | Specification | Purpose |
| --- | --- | --- |
| Camera | USB or IP, 1280x720 @ 30 fps minimum | Main vision sensor |
| CAN interface | USB-CAN adapter (Kvaser Leaf / PEAK PCAN-USB) | Steering controller bus |
| Steering controller | Accepts CAN frames on ID 0x123 | Executes turn/stop commands |
| ArUco markers | Printed at >= 20 cm x 20 cm, laminated | End-of-row triggers |
| Single-board PC | Raspberry Pi 5 or Jetson Nano (GPU optional) | Onboard compute |
| Power supply | 12 V regulated (5 V via USB for Pi) | Field power |

# Phase 2 — Camera Calibration

IMPORTANT: The default camera_matrix in config/default.yaml is a placeholder. You must calibrate your specific camera before deploying to the field.

### Step 1 — Print a calibration chessboard

Use OpenCV's 9x6 chessboard pattern. Print at A4 size and glue to a flat rigid board.

### Step 2 — Run the calibration script

NOTE: tools/calibrate_camera.py must be created before this step. Ask your developer to add it, or use any standard OpenCV chessboard calibration script.

python tools/calibrate_camera.py

Capture 20+ images from different angles by pressing SPACE. The script outputs:

camera_matrix:  [[fx, 0, cx], [0, fy, cy], [0, 0, 1]]

dist_coeffs:    [[k1, k2, p1, p2]]

### Step 3 — Update config/default.yaml

detectors:

aruco:

camera_matrix: [[fx, 0, cx], [0, fy, cy], [0, 0, 1]]

dist_coeffs: [[k1, k2, p1, p2]]

marker_length_m: 0.20     # physical marker size in metres

### Step 4 — Validate calibration

Reprojection error should be < 1.0 pixels. The calibration_validator.py module checks this.

# Phase 3 — CAN Bus Configuration

### Step 1 — Connect the USB-CAN adapter

Linux:

sudo ip link set can0 up type can bitrate 500000

sudo ip link set can0 txqueuelen 1000

Windows:

Install the adapter vendor driver (Kvaser: kvaser.com/drivers, PEAK: peak-system.com)

### Step 2 — Update config/default.yaml

controller:

can_interface: "socketcan"   # Linux: socketcan / Windows: kvaser or pcan

can_channel:   "can0"

can_bitrate:   500000

turn_command_id:   291       # 0x123 — must match your controller

estop_command_id:  0         # 0x000 — highest CAN priority

### Step 3 — Test CAN (loopback)

python -c "import can; bus = can.interface.Bus('can0', bustype='socketcan'); print('CAN OK')"

### Step 4 — Send a test E-STOP manually

python -c "

import can

bus = can.interface.Bus('can0', bustype='socketcan', bitrate=500000)

msg = can.Message(arbitration_id=0x000, data=[0xFF,0xFF,0,0,0,0,0,0])

bus.send(msg)

print('E-STOP sent')

bus.shutdown()"

Verify on the controller side (oscilloscope or CAN logger) that the frame was received.

# Phase 4 — ArUco Marker Deployment

### Generate a marker

python tools/generate_marker.py --id 0 --save

This saves aruco_marker_id0.png to the current directory.

### Print and mount guidelines

- Minimum size: 20 cm x 20 cm (physical marker face, not including white border)

- Material: laminated paper or aluminium composite panel

- Mount perpendicular to the robot's travel direction at each row end

- Height: centre of marker at camera height +/- 10 cm

- Marker ID 0 is reserved as the primary turn trigger

### Verify detection with live camera

python main.py

Walk toward the marker. Expect:

- Green bounding box appears at ~5 m

- Blue pose axes drawn over the marker

- Distance reading decrements in the dashboard

- STATE transitions to TURNING when distance <= 1.5 m

# Phase 5 — Integration Testing (Bench)

Before field deployment, do bench testing with the full hardware stack but with the robot stationary.

- Camera connects and streams at configured resolution/FPS

- ArUco marker detected correctly at known distance (measure with tape)

- Measured distance matches reported distance_m in dashboard (< 5% error)

- CAN bus initialises without error in logs

- U-TURN triggered by holding marker at < 1.5 m: CAN message observed on bus

- E-STOP triggered by disconnecting camera for > 1.5 s: E-STOP CAN message sent

- Auto-recovery fires when camera reconnects (enable_auto_recovery: true)

- YOLO correctly detects a person walking into frame

- Dashboard accessible from a separate device on the same network

# Phase 6 — Field Testing

## Pre-Field Checklist

- config/default.yaml reviewed and saved — no placeholder values remain

- camera_matrix and dist_coeffs updated from real calibration

- can_interface and can_channel match the hardware adapter

- recording.enabled: true — log all field sessions

- robot_id set to a unique ID (e.g. "robot-field-01")

- Battery / power supply charged

## Field Test Sequence

- Place markers at row ends

- Start the system:  python main.py --robot-id robot-field-01

- Monitor dashboard from a phone/tablet browser on field WiFi

- Drive one row manually at low speed — confirm STATE stays DRIVING

- Approach a row-end marker — confirm STATE transitions to TURNING and turn executes

- Confirm robot straightens and resumes DRIVING

- Walk in front of the robot — confirm STATE transitions to STOPPED immediately

- Walk away — confirm STATE resumes (if auto_recovery: true)

- Press Q or Ctrl-C to end session

- Review the auto-generated test report in recordings/

## Tuning Guide

| Symptom | Parameter | Direction |
| --- | --- | --- |
| Steering oscillates | navigation.smoothing_alpha | Decrease (e.g. 0.1) |
| Slow to react to alignment | navigation.smoothing_alpha | Increase (e.g. 0.5) |
| Turns too early | detectors.aruco.turn_trigger_distance_m | Decrease |
| Turns too late | detectors.aruco.turn_trigger_distance_m | Increase |
| False obstacle stops | detectors.yolo.danger_zone_ratio | Increase (e.g. 0.2) |
| Misses real obstacles | detectors.yolo.danger_zone_ratio | Decrease (e.g. 0.08) |
| Too many stale-frame stops | safety.max_stale_frame_ms | Increase (e.g. 3000) |
| Marker lost during turn | navigation.recovery_timeout_s | Increase (e.g. 5.0) |

TIP: All tuning changes can be made live by editing config/default.yaml — the file watcher reloads within 2 seconds without restarting the application.

# Phase 7 — Ongoing Monitoring

| Tool | How to Access | What to Watch |
| --- | --- | --- |
| Web dashboard | http://<robot-ip>:5000 | State, FPS, obstacles |
| Prometheus | http://<robot-ip>:5000/metrics | Scrape with Prometheus + Grafana |
| Runtime log | logs/runtime.log | All INFO+ events |
| Navigation log | logs/navigation.log | State transitions and CAN commands |
| Safety log | logs/safety.log | E-STOP events and camera faults |
| Session report | recordings/<session>/test_report.json | Post-session statistics |

APPENDIX A — QUICK REFERENCE COMMANDS

Run all unit tests:

python -m pytest tests/ -v

Start simulation (no hardware):

python main.py --simulate

Start with live camera:

python main.py

Start with specific robot ID:

python main.py --robot-id robot-01

Replay a recorded session:

python main.py --replay recordings/<session> --speed 1.0

Run stress test:

python main.py --simulate --stress

Check linting:

python -m flake8 .

Check types:

python -m mypy . --ignore-missing-imports

Generate ArUco marker:

python tools/generate_marker.py --id 0 --save

Collect training data:

python tools/collect_training_data.py

Train custom YOLO model:

python training/train.py --epochs 100 --device cuda

APPENDIX B — TROUBLESHOOTING

| Problem | Likely Cause | Fix |
| --- | --- | --- |
| Camera not found | Wrong index or blocked by another app | Run: python tools/find_camera.py |
| ArUco not detected | Bad calibration or wrong dictionary name | Re-calibrate; check detectors.aruco.dictionary |
| CAN bus error at init | Wrong interface or channel name | Check config and driver installation |
| YOLO not loading | ultralytics not installed or bad model path | pip install ultralytics; check model path |
| Dashboard not loading | Port 5000 blocked or Flask not installed | pip install flask; check firewall |
| Frequent stale-frame stops | Camera USB bandwidth issue or lag | Increase safety.max_stale_frame_ms |
| 66 tests pass but app crashes | Missing runtime dependency | pip install -r requirements.txt |
| mypy errors in new code | Missing Optional or type annotation | Follow patterns in existing fixed files |