AgriBot Vision System

How to Start & Use the Application

Simple step-by-step guide — no programming knowledge needed

E:\TTS\Projects\Web-Applications\Image-processing

What Does This App Do?

This application uses your computer's camera to watch for special markers placed at the end of crop rows on a farm.

When the robot gets close to a marker, the app tells the robot to turn around and go back up the next row — fully automatically.

📷  It shows you a live video window on screen so you can see exactly what the camera sees.

🌐  It also opens a webpage (like a control panel) that you can view from your phone or any browser.

⚠️  If something dangerous (like a person) walks in front of the robot, it automatically stops.

Before You Start — Check This First

Things you need:

| What | Details |
| --- | --- |
| A Windows computer | The folder is already set up on your PC |
| A USB camera (optional) | Plug it in before starting the app. A regular webcam works fine. |
| The app folder | E:\TTS\Projects\Web-Applications\Image-processing |
| Python installed | Already installed — nothing to do here |

✅  No camera? No problem! You can run the app in Simulation mode — it will pretend a camera is there and show you fake markers moving around so you can see how it works.

How to Open the Black Command Window

The app is started by typing a simple command into a black window called the Terminal (or Command Prompt). Here is how to open it:

STEP 1   Right-click on the Start button

Click the Windows logo in the bottom-left corner of your screen using your RIGHT mouse button.

STEP 2   Click "Terminal" or "Command Prompt"

A small menu pops up. Look for "Terminal", "Windows Terminal", or "Command Prompt" and click on it.

💡  Tip: If you see both "Terminal" and "Terminal (Admin)" — just click plain "Terminal". No admin needed.

STEP 3   A black window opens

You will see a black (or dark blue) window with some text and a blinking cursor. That is normal — this is where you type commands.

Step 1 — Go to the App Folder

In the black window, type the following and press ENTER:

cd E:\TTS\Projects\Web-Applications\Image-processing

After pressing ENTER you will see the folder path appear at the start of the next line. That means you are in the right place.

💡  Think of this like opening a folder on your computer — you are telling the computer which folder to work in.

Step 2 — Choose How to Start

There are two ways to run the app. Pick one:

| Mode | When to use it | What you need |
| --- | --- | --- |
| SIMULATION MODE (Recommended to try first) | Test the app without a real camera. Good for learning how it works. | Nothing extra needed |
| LIVE CAMERA MODE | Use a real USB camera to detect actual markers. | A USB webcam plugged in |

Option A — Start Without a Camera (Simulation)

Type this command exactly and press ENTER:

python main.py --simulate

After a few seconds, TWO things will happen:

- A video window opens on your screen showing a moving marker — this is the fake camera feed.

- A website starts running on your computer. You can open it in any browser (see Section 5 below).

✅  If you see a window with coloured boxes and text on the screen, it is working! You will see the word "DRIVING" at the top. The app is running.

❌  If you see red error text, check that you typed the command correctly and pressed ENTER.

Option B — Start With a Real Camera

STEP 1   Plug in your USB camera

Connect your USB webcam to any USB port on your computer. Wait 5 seconds for Windows to recognise it.

STEP 2   Check Windows can see the camera

Open the Windows Camera app (search for "Camera" in the Start menu). If you can see a live image, the camera is working and you can continue.

⚠️  IMPORTANT: Close the Windows Camera app before starting AgriBot. Two apps cannot use the same camera at the same time.

STEP 3   Start the app

In the black terminal window, type this and press ENTER:

python main.py

✅  A video window should open showing what your camera sees. Coloured boxes will appear around any detected markers.

Camera not working? Try these fixes:

| Problem | What to try |
| --- | --- |
| Error: No cameras found | Make sure the camera app (Windows Camera) is CLOSED first, then try again |
| Black screen in the video window | Unplug the camera, wait 5 seconds, plug it back in, then run the command again |
| App starts but camera looks wrong | Run: python tools/find_camera.py — it will find the right camera number for you |

What You Will See on Screen

When the app is running, you will see a video window with information overlaid on top of the camera image:

| What you see | What it means |
| --- | --- |
| STATE: DRIVING | The robot is moving forward normally. Everything is fine. |
| STATE: TURNING | The robot spotted a marker and is executing a U-turn. |
| STATE: STOPPED | Something blocked the path — the robot has stopped for safety. |
| STATE: RECOVERING | A marker was spotted but then disappeared. The robot is waiting. |
| Green box with a number | An ArUco marker has been detected and identified. |
| Blue X-Y-Z lines on the marker | The app has measured the exact distance and angle to the marker. |
| Red box around something | An obstacle (person, animal, vehicle) detected — robot is stopping. |
| Blue arrow at the bottom | Shows which direction the steering is correcting towards. |
| FPS number | Frames Per Second — how fast the camera is processing. 20+ is good. |
| STABILITY score | How steady the marker detection is. Higher = better. |

💡  Press the letter Q on your keyboard to quit the app cleanly when you are done.

The Web Dashboard — Your Control Panel

While the app is running, it also creates a live webpage you can view from any browser — even from your phone on the same WiFi network!

STEP 1   Open any web browser

Open Google Chrome, Microsoft Edge, Firefox — any browser works.

STEP 2   Go to this address

Click in the address bar at the top of the browser, type this, and press ENTER:

http://localhost:5000

💡  If viewing from your phone: replace "localhost" with your computer's IP address. Example: http://192.168.1.5:5000

STEP 3   You will see the live dashboard

The dashboard shows:

| Dashboard section | What it shows |
| --- | --- |
| Live Camera Feed | The same video the robot is seeing — updates in real time |
| Vehicle State | Current status: DRIVING, TURNING, STOPPED, etc. |
| FPS | How fast the camera is processing (frames per second) |
| Latency | How quickly the app responds to what it sees (lower = better) |
| Steering | Which direction the robot is being guided |
| Distance | How far away the nearest marker is, in metres |
| Detected Obstacles | List of any dangerous objects currently in view |
| System Log | A live list of what the app is doing, updated every second |
| FPS & Latency Chart | A graph showing performance over the last 60 seconds |

✅  The connection dot at the top-left of the dashboard is your health indicator: green = all good, yellow = turning, red = stopped or disconnected.

Testing With a Real Marker

Want to see the detection actually work? Here is how to test it right now with a printed marker:

STEP 1   Print the marker

Run this in the black terminal window:

python tools/generate_marker.py --id 0 --save

This saves a file called "aruco_marker_id0.png" in the app folder. Open it and print it on A4 paper. Print it as large as possible — at least 15 cm x 15 cm.

STEP 2   Hold it in front of the camera

Hold the printed marker in front of the camera. Try different distances — from about 30 cm away to about 2 metres away.

STEP 3   Watch what happens

On the video window you should see:

- A green box appears around the marker

- Text shows the marker ID ("ID:0") and the distance in metres

- Blue lines (like a 3D cross) appear on the marker showing its angle

- When you get closer than 1.5 metres, the STATE changes to TURNING

✅  If all of the above happens, the application is working perfectly! The camera is detecting markers just as it would in the field.

How to Stop the Application

| To stop the app | Do this |
| --- | --- |
| Normal stop | Click on the video window and press the letter Q on your keyboard |
| Emergency stop | Press Ctrl and C at the same time in the terminal window |
| If nothing works | Close the terminal window — this forces the app to stop |

💡  After stopping, the app automatically saves a report of your session in a folder called "recordings" — only if recording was turned on in the settings.

Quick Reference — All Commands at a Glance

| What you want to do | Type this command, then press ENTER |
| --- | --- |
| Go to the app folder first (always do this first!) | cd E:\TTS\Projects\Web-Applications\Image-processing |
| Start app WITHOUT a camera (simulation) | python main.py --simulate |
| Start app WITH a live camera | python main.py |
| Find out which camera number to use | python tools/find_camera.py |
| Print an ArUco test marker | python tools/generate_marker.py --id 0 --save |
| Stop the app | Press Q in the video window, or Ctrl+C in the terminal |

Something Not Working? Check Here First

| Problem | Simple fix |
| --- | --- |
| The black terminal window shows red text | The command was typed wrong. Check spelling carefully. Remember capitals matter — type python not Python. |
| "No module named" error appears | A software package is missing. Ask your developer to run: pip install -r requirements.txt |
| The video window opens but is black | Your camera may be blocked by another app. Close the Windows Camera app and try again. |
| The webpage at localhost:5000 says "This site can't be reached" | The app may still be starting up — wait 5 seconds and refresh the page. If still not working, make sure the terminal window is still open and running. |
| Markers are not detected | Make sure the marker is printed large enough (at least 15 cm), the lighting is good (not too dark or too bright), and the marker is held flat and straight in front of the camera. |
| The app says "STOPPED" and stays there | Something is in front of the camera triggering the obstacle detector. Remove it from view. The robot resumes automatically. |
| Everything freezes | Close the terminal window and start again from Step 1. |

📞  Still stuck? The black terminal window always shows a log of what happened. Take a photo of it and send it to your developer — it tells them exactly what went wrong.