AgriBot Vision System

Complete Feature Guide

How every feature works — in plain English

All values shown are live defaults from config/default.yaml

🌾  Overview — What This System Does

The AgriBot Vision System is a software brain for an agricultural robot. It watches the world through a camera, understands what it sees, and tells the robot what to do — all automatically, 30 times per second.

It has three main jobs:

- Follow crop rows in a straight line by keeping a special marker centred in the camera

- Turn around at the end of each row when it gets close enough to the end-of-row marker

- Stop instantly if a person, animal, or large vehicle gets in the way

| Feature area | What it does |
| --- | --- |
| ArUco Marker Detection | Reads special black-and-white square markers placed at row ends |
| Obstacle Detection (YOLO) | Detects people, animals, and vehicles in real time using AI |
| Steering Correction | Keeps the robot centred on the row using smooth, filtered corrections |
| State Machine | Manages robot states: DRIVING, TURNING, RECOVERING, STOPPED, IDLE |
| Safety Monitor | Emergency-stops the robot if the camera feed dies or a danger appears |
| Web Dashboard | Shows live video, status, logs, and performance graphs in a browser |
| Recording & Replay | Saves every session to disk so you can replay and debug it later |
| Config Hot-Reload | Let you change settings while the robot is running — no restart needed |

🎯  Feature 1 — ArUco Marker Detection (Row-End Sensing)

What is an ArUco marker?

An ArUco marker is a special square black-and-white image — similar to a QR code — printed on paper or board and placed at the end of each crop row. The robot's camera reads this marker to know it has reached the end of the row.

📐  The marker used is from the DICT_6X6_250 dictionary — meaning it is a 6×6 grid pattern, and there are up to 250 unique designs. Each has a unique ID number (0, 1, 2 … 249).

What exactly does the camera measure?

For every detected marker, the app calculates ALL of these in real time:

| Measurement | What it tells the robot | How it's used |
| --- | --- | --- |
| Distance (m) | How far away the marker is, in metres | Decides when to trigger a U-turn |
| Center X position (px) | How far left or right the marker is in the frame | Steers the robot to stay centred |
| Yaw angle (°) | How much the marker is rotated left or right | Orientation feedback |
| Pitch angle (°) | How much the marker tilts up or down | Approach angle feedback |
| Roll angle (°) | How much the marker is rolling sideways | Structural integrity check |
| Confidence score | How reliably the marker was detected (0–1) | Filters out false detections |
| Blur score | How sharp the camera image is | Quality warning if too blurry |

How does the U-turn get triggered?

Turn trigger distance:  1.5 metres

When the marker's measured distance drops to 1.5 metres or less, the system flags that marker as a turn-trigger. On the very next detection frame, the robot transitions to TURNING state and a U-TURN command is sent over the CAN bus to the steering controller.

🔄  After a turn is triggered, a cooldown of 10 seconds is enforced before the next turn can happen — this prevents the robot from accidentally triggering multiple turns in quick succession.

What if the marker disappears just before the turn?

This is handled by the Recovery Manager. Here is what happens step by step:

- Robot is tracking a close marker (inside 1.5 m) — turn-trigger visible

- Marker suddenly vanishes from camera view (blocked, lighting change, etc.)

- Robot transitions to RECOVERING state immediately — it does NOT stop

- The system waits up to 2 seconds for the marker to reappear

- If the marker comes back in those 2 seconds — the turn proceeds normally

- If it does NOT come back after 2 seconds — robot transitions to STOPPED

Recovery timeout:  2.0 seconds before STOPPED

✅  Normal forward driving without any marker in view is perfectly fine — the Recovery logic only activates when a marker was actively being tracked and then disappeared.

Can it detect markers from an angle?

Yes. The detector uses a fallback strategy:

- First, it tries the configured dictionary (DICT_6X6_250)

- If no markers found, it automatically tries DICT_6X6_50, DICT_6X6_100, and DICT_6X6_1000 as fallbacks

- This means even if the wrong dictionary was configured, the marker will still be found

Detection parameters are also tuned for maximum robustness — the detector uses an adaptive threshold window that ranges from 3px to 23px to handle different lighting and distances.

Can it use barcodes or other image types?

Yes — the system has three detector types for markers:

| Detector type | How it works | Current status |
| --- | --- | --- |
| ArUco (primary) | Reads square black-and-white fiducial markers. Most accurate — gives exact distance and angle. | ENABLED by default |
| Barcode / QR code | Reads standard barcodes and QR codes printed at row ends using the pyzbar library. | Available — disabled by default. Enable in config. |
| Feature matching | Matches any custom image template (e.g. a photo or logo) using ORB keypoint detection. | Available — disabled by default. Needs a template image. |

💡  To enable barcodes: in config/default.yaml set detectors.barcode.enabled: true. To enable feature matching: set detectors.feature.enabled: true and provide a template image file.

🧭  Feature 2 — Steering Correction (Keeping the Robot Centred)

How does it steer?

The camera frame is 1280 pixels wide. The centre of the frame is pixel 640. Wherever the ArUco marker sits horizontally in the frame determines the steering correction:

| Marker position in frame | Steering correction | What the robot does |
| --- | --- | --- |
| Marker is at centre (640 px) | Zero correction | Drive straight — no adjustment needed |
| Marker is to the right (> 660 px) | Positive correction | Steer right to re-centre the marker |
| Marker is to the left (< 620 px) | Negative correction | Steer left to re-centre the marker |

Dead zone — why it doesn't jitter on straight roads

Dead zone width:  ±20 pixels from centre

If the marker is within 20 pixels of the frame centre (between pixel 620 and 660), the steering correction is set to exactly zero. This dead zone prevents the robot from making tiny, jittery steering adjustments when it is already driving straight — which would cause wobbling.

Smoothing — why steering feels gradual

Smoothing factor (alpha):  0.3 (0 = very smooth, 1 = instant)

Raw steering values jump around a lot — the camera sees the marker slightly differently in each frame. To prevent harsh, jerky corrections, the system uses an Exponential Moving Average (EMA) filter. With alpha=0.3, each new correction is blended: 30% new value + 70% previous value. This makes steering feel smooth and gradual, like a human driver.

⚙️  If the robot reacts too slowly to marker movement, increase smoothing_alpha (e.g. 0.5). If steering oscillates or wobbles, decrease it (e.g. 0.1). Change it live in config/default.yaml — no restart needed.

🛑  Feature 3 — Obstacle Detection (YOLO AI Safety System)

What is YOLO and how does it work?

YOLO (You Only Look Once) is an AI model that has been trained on millions of images. It looks at the entire camera frame at once and draws bounding boxes around every object it recognises — in real time. This app uses YOLOv8 Nano (the smallest, fastest version).

The model runs in a background thread so it never slows down the main ArUco detection pipeline. Every new frame is passed to the YOLO thread, and the most recent results are displayed without waiting.

What objects can it detect?

The system detects 11 types of objects that could be hazards on a farm:

| Category | Objects Detected |
| --- | --- |
| People | Person (any person — worker, bystander, child) |
| Vehicles | Bicycle, Car, Motorcycle, Bus, Truck |
| Farm animals | Cat, Dog, Horse, Sheep, Cow |

🔬  Minimum confidence to report a detection: 50%. Objects detected below 50% confidence are silently ignored — this reduces false alarms from shadows, reflections, or partially visible objects.

Does it see obstacles on the SIDES of the machine?

The camera has a field of view that is 1280 pixels wide. YOLO scans the ENTIRE frame — from the leftmost to the rightmost pixel. So yes, objects at the edges of the frame (to the left or right of the robot's path) ARE detected and shown with a bounding box.

However — whether a side object STOPS the robot depends on its size in the frame:

Critical zone threshold:  12% of total frame area

| Object size in frame | What happens |
| --- | --- |
| Smaller than 12% of frame area | Object is detected and shown in orange — logged as present. Robot does NOT stop. |
| 12% or larger of frame area | Object is flagged CRITICAL — red box shown, robot immediately STOPS. |

In practical terms: the frame is 1280×720 = 921,600 pixels total. The 12% threshold means an object must cover at least 110,592 pixels (roughly a 330×330 pixel box) before it triggers an emergency stop.

At what distance does the robot stop?

There is no fixed distance — the stopping trigger is based on how much of the camera frame the obstacle fills, not a measured distance. This is because:

- A large truck far away can fill the same area as a person close up

- Area-based detection is more relevant than distance for safety purposes

However, as a practical estimate for a typical adult person (roughly 60 cm wide × 170 cm tall):

| Person distance from camera | Approximate frame coverage | Robot reaction |
| --- | --- | --- |
| 3+ metres away | < 5% of frame | Detected, shown in orange, robot keeps moving |
| ~1.5–2 metres away | ~8–11% of frame | Detected, shown in orange, robot keeps moving |
| ~1.0–1.2 metres away | ~12%+ of frame | CRITICAL — robot STOPS immediately |
| < 0.5 metres away | 30%+ of frame | CRITICAL — robot already stopped |

⚙️  To make the robot stop earlier (more cautious): decrease danger_zone_ratio to e.g. 0.08 (8%). To stop later (less sensitive): increase to e.g. 0.20 (20%). Change it live in config/default.yaml.

What exactly happens when an obstacle is detected?

The sequence is precise and instant:

- YOLO detects a person (or other object) filling ≥ 12% of the frame

- The detection is flagged CRITICAL — a red bounding box is drawn on screen

- The text '!! OBSTACLE DETECTED: PERSON !!' appears in the video window

- STATE immediately changes from DRIVING to STOPPED

- An E-STOP command (CAN message ID 0x000) is sent to the steering controller — 3 attempts with retry

- The steering correction goes to zero — no steering output while stopped

- The dashboard shows the obstacle in the Detected Obstacles panel with a red CRITICAL badge

What happens when the obstacle moves away?

The robot resumes AUTOMATICALLY — no button press needed:

- YOLO no longer detects a critical obstacle in the frame

- STATE immediately transitions back to DRIVING

- Steering correction resumes — robot continues along the row

✅  The robot does not need a human to press a button to resume. As soon as the path is clear, it starts moving again. This is the expected behaviour for autonomous agricultural operation.

What if YOLO stops working?

The system is designed to fail safely. If YOLO has 5 consecutive inference errors (e.g. GPU crash, model file corruption), it:

- Disables itself and logs CRITICAL: 'YOLO disabled after 5 consecutive errors'

- The robot continues driving and detecting ArUco markers without obstacle detection

- The dashboard shows a yellow warning banner: 'YOLO obstacle detection DISABLED'

- The Prometheus metric agribot_yolo_faulted changes to 1

⚠️  This is a deliberate safety trade-off: running without obstacle detection is safer than stopping the entire robot due to a software crash. An operator monitoring the dashboard should see the warning and take over.

🔄  Feature 4 — The State Machine (Robot Brain Logic)

The robot is always in exactly one of five states. The state determines what the robot does and what decisions it can make:

| State | What it means | How to get here | Steering output |
| --- | --- | --- | --- |
| IDLE | Just started, not yet moving | System startup | None |
| DRIVING | Moving forward, scanning for markers and obstacles | System start, obstacle cleared, turn complete | Active — marker-guided |
| TURNING | Executing a U-turn at the row end | Marker inside 1.5 m trigger distance | Controlled by CAN command |
| RECOVERING | Marker just disappeared — waiting for it to return | Marker lost while inside trigger distance | Straight ahead (no correction) |
| STOPPED | Emergency halt — obstacle or timeout | Critical obstacle OR recovery timeout exceeded | None — zero output |

State transition rules

The state machine has built-in protection against rapid state flickering:

Minimum time in any state:  0.5 seconds before switching

Exception: STOPPED can always be entered immediately regardless of how long the robot has been in its current state. Safety always overrides the minimum duration rule.

Priority order (highest wins):

- CRITICAL OBSTACLE detected → STOPPED immediately (overrides everything)

- Turn trigger marker at ≤ 1.5 m → TURNING

- Marker lost after tracking close trigger → RECOVERING

- Recovery timeout exceeded (2 s) → STOPPED

- Obstacle cleared → DRIVING

- Marker re-acquired → DRIVING

🛡️  Feature 5 — Safety Monitor (Camera Watchdog)

Separate from obstacle detection, the Safety Monitor watches for a different kind of failure: what if the camera cable gets unplugged, or the USB connection drops? Without this, the robot would keep driving blind.

Max allowed gap between frames:  1,500 ms (1.5 seconds)

The safety monitor checks whether a new camera frame arrived within the last 1.5 seconds. If it didn't:

- It logs: SAFETY VIOLATION: Stale frame detected

- It sends an E-STOP command to the CAN bus (3 attempts with retry)

- The robot halts

Auto-recovery from camera reconnection

When auto-recovery is turned on (the default), the robot does NOT stay stopped forever if the camera comes back:

- Camera reconnects — new frames start arriving again

- Safety Monitor detects the camera is alive again

- E-STOP state is automatically cleared

- Robot resumes DRIVING

⚙️  To turn off auto-recovery (fail-safe mode for unattended operation): set safety.enable_auto_recovery: false in config. The robot will stay stopped until manually restarted.

📊  Feature 6 — Web Dashboard (Remote Monitoring)

While the robot is running, a webpage is available at http://localhost:5000 (or http://<robot-ip>:5000 from a phone). It updates automatically every 0.5 seconds.

| Panel | What it shows | Update rate |
| --- | --- | --- |
| Live Camera Feed | Exact video the robot sees — with all overlays (boxes, state text, steering arrow) | ~30 fps (MJPEG stream) |
| Vehicle State | Current state in colour: green=DRIVING, yellow=TURNING, red=STOPPED | Every 0.5 s |
| FPS | How many camera frames are processed per second (target: 20–30) | Every 0.5 s |
| Latency | How long each frame takes to process in milliseconds (lower = better) | Every 0.5 s |
| Steering | Current steering correction value (positive = right, negative = left) | Every 0.5 s |
| Distance to target | Metres to the nearest ArUco marker (or '—' if none visible) | Every 0.5 s |
| Detected Obstacles | List of all currently visible obstacles with confidence % and CRITICAL badge | Every 0.5 s |
| System Log | Live scrolling log of every event — colour-coded by severity | Every 0.5 s |
| FPS & Latency chart | Line graph of the last 60 seconds of performance history | Every 0.5 s |
| Uptime | How long the system has been running (HH:MM:SS) | Every 0.5 s |
| YOLO fault banner | Yellow warning if YOLO obstacle detection has disabled itself | Every 0.5 s |

Prometheus monitoring (/metrics endpoint)

For advanced users with Grafana/Prometheus, the app also exposes a machine-readable metrics page at http://<robot-ip>:5000/metrics with counters for FPS, latency, E-STOP count, obstacle count, uptime, and more.

🎬  Feature 7 — Session Recording & Replay

What gets saved?

When recording is enabled in the config, every session automatically saves:

| File/Folder | Contents |
| --- | --- |
| raw/ | Original camera frames as .jpg files — what the camera actually saw |
| annotated/ | The same frames with all overlays drawn (bounding boxes, state text, arrows) |
| session.json | Per-frame metadata: timestamp, state, steering, detected targets, obstacles |
| test_report.json | Summary statistics: detection rate, average FPS, E-STOP count, session duration |

Replay mode

Any recorded session can be replayed through the full pipeline as if the camera were live. This lets you:

- Debug why the robot stopped unexpectedly — replay it at 0.5× speed

- Test config changes against real recorded data — no need to go to the field

- Show a stakeholder exactly what the robot saw during a session

🎛️  Replay speed can be controlled with --speed: 0.5 = half speed (slow motion), 1.0 = real time, 2.0 = double speed (fast-forward). Example: python main.py --replay recordings/session1 --speed 0.5

⚙️  Feature 8 — Live Configuration (Hot-Reload)

Every tunable parameter in the system can be changed while the robot is running. There is no need to stop and restart the app.

How it works: the app watches config/default.yaml for changes. The moment you save the file, the app detects it within 2 seconds and applies the new values to all running components immediately.

| Parameter | What it controls | Default value |
| --- | --- | --- |
| navigation.smoothing_alpha | How smooth or responsive the steering is | 0.3 |
| navigation.dead_zone_x | How many pixels of 'wobble' are ignored (prevents jitter) | 20 px |
| navigation.recovery_timeout_s | How long to wait for a lost marker before stopping | 2.0 s |
| navigation.turn_cooldown_s | Minimum time between two consecutive turns | 10.0 s |
| navigation.min_state_duration_s | Minimum time in any state before switching | 0.5 s |
| detectors.aruco.turn_trigger_distance_m | Distance at which a U-turn is triggered | 1.5 m |
| detectors.yolo.danger_zone_ratio | Fraction of frame that triggers emergency stop | 0.12 (12%) |
| detectors.yolo.confidence | Minimum AI confidence to count a detection | 0.50 (50%) |
| safety.max_stale_frame_ms | Max allowed gap between camera frames before E-STOP | 1500 ms |
| safety.enable_auto_recovery | Whether to auto-resume after a stale-frame E-STOP | true |

📋  Quick Summary — All Key Numbers at a Glance

| Question | Answer |
| --- | --- |
| At what distance does it trigger a U-turn? | 1.5 metres from the ArUco marker |
| How many times per second does it process frames? | 30 frames per second (camera runs at 30 fps) |
| At what distance does it stop for a person? | Approx. 1.0–1.2 m (when person fills 12% of frame) |
| Does it detect obstacles on the sides? | Yes — full 1280px wide frame is scanned. Stops only if obstacle fills ≥12% of frame. |
| What does it detect as an obstacle? | Person, bicycle, car, motorcycle, bus, truck, cat, dog, horse, sheep, cow |
| Does it resume automatically after obstacle clears? | Yes — as soon as the path is clear, it drives again. No button needed. |
| How long does it wait if the camera dies? | 1.5 seconds, then E-STOP fires |
| Does it auto-recover if camera reconnects? | Yes (by default). Disabled in fail-safe mode. |
| How long between two allowed U-turns? | 10 seconds cooldown |
| How long does it wait for a lost marker? | 2 seconds (RECOVERING), then STOPPED |
| Can you change settings without restarting? | Yes — edit config/default.yaml, changes apply within 2 seconds |
| Can it detect barcodes or QR codes too? | Yes — enable detectors.barcode.enabled: true in config |
| Can it use a custom image as a row-end marker? | Yes — enable feature detector in config and provide a template image |
| What happens if YOLO crashes? | YOLO disables itself after 5 errors. Robot continues without obstacle detection. |
| How many markers can it track at once? | Multiple — the closest one with the highest priority is used for steering |