AgriBot Vision System

Sensor Calibration Guide

Camera  -  LiDAR  -  Radar  -  GPS  -  Sensor Alignment

Overview - Which Calibrations You Need

Calibration tells the software the exact properties and position of each sensor, so the numbers it reads (a marker's distance, a LiDAR point, a GPS fix) map correctly to the real world. Do them in the order below — each later step assumes the earlier ones are done.

| # | Calibration | Needs hardware? | Done when |
| --- | --- | --- | --- |
| 1 | Camera intrinsic (lens) | Camera only | Reprojection error < 1.0 px |
| 2 | Camera mounting (extrinsic) | Camera + tape measure | Height & tilt recorded in config |
| 3 | LiDAR mounting & zero | LiDAR | Ground plane reads ~0; range matches tape |
| 4 | Camera <-> LiDAR alignment | Camera + LiDAR | A marker's camera & LiDAR position agree |
| 5 | Radar mounting & offset | Radar | Known target range/speed matches |
| 6 | GPS reference & heading | GPS (RTK ideal) | Local X/Y origin set; heading aligned |
| 7 | IMU / compass (optional) | IMU | Heading drift acceptable; mag calibrated |
| 8 | Wheel / steering odometry | Drive base | Commanded vs measured distance matches |

1. Camera Intrinsic (Lens) Calibration

Purpose: measure the camera's focal length and lens distortion so marker distances are accurate (this controls when the U-turn fires).

You need

- A printed 9x6 inner-corner chessboard (10x7 squares) on A4, glued flat to a rigid board.

- The camera you will actually use, at its final resolution.

Steps (tool already provided)

python tools/calibrate_camera.py

- Hold the board in view; green corners appear when detected.

- Press SPACE to capture 15-20 samples from varied angles, tilts and distances.

- Press C to calibrate. The tool prints + saves camera_calibration.yaml.

- Already have photos? Use offline mode: python tools/calibrate_camera.py --images <folder>

Where the result goes

detectors:

aruco:

camera_matrix: [[fx,0,cx],[0,fy,cy],[0,0,1]]

dist_coeffs:   [[k1,k2,p1,p2,k3]]

PASS  Acceptance: reprojection error < 1.0 pixel. If higher, recapture with more varied angles and good lighting.

2. Camera Mounting (Extrinsic) Calibration

Purpose: tell the software where the camera sits on the machine and how it is tilted, so image positions map to ground positions in front of the machine.

Measure and record

| Quantity | How to measure | Typical |
| --- | --- | --- |
| Mount height | Tape from ground to lens centre | 1.0 - 1.8 m |
| Tilt (pitch) down | Angle gauge / phone level on the camera | 10 - 25 deg |
| Forward offset | Tape from drive axle to lens | machine-specific |
| Lateral offset | Tape from machine centre-line to lens | 0 (centre) ideal |

TIP  Mount the camera on the machine centre-line, facing straight forward, tilted slightly down so the row ahead fills the frame. Record these numbers — the LiDAR/radar alignment (steps 4-5) reference the same machine frame.

3. LiDAR Mounting & Zero Calibration

Purpose: the LiDAR replaces fragile vision for detecting the 1-metre end-of-row poles and for obstacle distance. Calibration fixes its position, orientation, and range zero.

Steps

- Mount the LiDAR rigidly, level, with a clear forward view. Record its height and forward/lateral offset (same machine frame as the camera).

- Range zero: place a flat target at a tape-measured distance (e.g. 2.000 m). Confirm the LiDAR reports the same; record any constant offset.

- Ground/horizon: on flat ground, confirm ground returns sit at the expected height/angle. Set a ground-removal threshold so the floor is ignored.

- Pole detection threshold: stand a 1 m pole at 1, 3, 5 m. Record the cluster width/height it produces so the detector can recognise 'a vertical ~1 m object'.

Config (planned keys)

sensors:

lidar:

mount_height_m: 0.8

forward_offset_m: 0.5

range_offset_m: 0.0

pole_min_height_m: 0.6   # accept poles >= this

pole_max_width_m: 0.25   # reject wide objects

PASS  Acceptance: a tape-measured distance matches the LiDAR reading within a few cm, and a 1 m pole is detected reliably at your turn-trigger distance.

4. Camera <-> LiDAR Alignment (Extrinsic)

Purpose: put camera detections and LiDAR detections into ONE shared coordinate frame, so 'the pole the camera sees' and 'the pole the LiDAR sees' are the same object.

Method

- Place a tall, thin target (a pole or board) a few metres ahead, visible to BOTH sensors.

- Record the target's position as seen by the camera and by the LiDAR.

- Compute the rigid transform (rotation + translation) that maps LiDAR points into the camera frame (or both into the machine frame).

- Repeat at several positions/distances and average.

INFO  In practice you store one 4x4 transform (or simple x/y/yaw offsets if both are roughly level and forward-facing). Keep both sensors on the same rigid bar to minimise this offset.

sensors:

lidar:

to_camera: { dx_m: 0.0, dy_m: -0.2, dyaw_deg: 0.0 }

5. Radar Mounting & Offset Calibration

Purpose: radar adds robust obstacle range AND closing speed in dust, fog, rain and low light where camera/LiDAR struggle. Calibration fixes its mounting and verifies range/velocity.

Steps

- Mount level and forward-facing; record height and offsets (same machine frame).

- Range check: a strong reflector (corner reflector / metal plate) at a tape-measured distance should match the radar range.

- Velocity check: walk toward/away at a steady pace; confirm the sign and rough magnitude of the reported closing speed.

- Set the obstacle thresholds: minimum range to trigger a slow-down vs a full stop.

sensors:

radar:

mount_height_m: 0.6

range_offset_m: 0.0

stop_range_m: 2.0     # full stop inside this

slow_range_m: 5.0     # slow down inside this

INFO  Radar gives range + speed but poor shape; LiDAR gives shape + precise range; the camera gives identity (what it is). Fusing all three is the most robust obstacle system.

6. GPS Reference & Heading Calibration

Purpose: GPS drives the farm-map coverage (the vacuum-style plan). Calibration sets the local origin and aligns the machine's heading to true north so map waypoints line up with reality.

Steps

- Reference point: park at a known fixed spot (e.g. the field's centre or a surveyed corner) and capture its lat/lon. This becomes local (0,0).

- The software converts all lat/lon to local X (Easting) / Y (Northing) metres from this point (already implemented in sensors/gps_receiver.py).

- Antenna offset: record where the GPS antenna sits relative to machine centre.

- Heading alignment: drive a straight, known line; compare GPS track heading to the machine's forward axis. Record the offset (or use dual-antenna / IMU for live heading).

- Accuracy: plain GPS is +/- 2-5 m (too coarse for rows). Use RTK / RTK-NTRIP for cm-level accuracy needed for row-following.

sensors:

gps:

port: "/dev/ttyUSB0"

reference_lat: 0.0     # set to your field origin

reference_lon: 0.0

heading_offset_deg: 0.0

use_rtk: true

WARN  Match the government farm map's coordinate system (datum, e.g. WGS84) to the GPS. A datum mismatch shifts the whole map by metres.

7. IMU / Compass Calibration (Optional)

Purpose: an IMU gives smooth, fast heading between GPS updates and during turns. The magnetometer (compass) needs calibration because the metal machine distorts the magnetic field.

- Hard-iron / soft-iron: rotate the machine slowly through full circles (figure-8) to calibrate the magnetometer offsets.

- Mount the IMU away from motors and large steel; record its orientation relative to the machine axes.

- Fuse IMU heading with GPS track (complementary / Kalman filter) for stable heading.

8. Wheel / Steering Odometry Calibration

Purpose: needed for the U-turn arc and for dead-reckoning between GPS fixes. Calibrates distance-per-wheel-tick and the steering response.

- Distance: command the machine to drive a tape-measured 10 m; compare to the wheel-encoder distance; record the scale factor.

- Steering: command known steering angles; measure the actual turning radius; build the command->radius map.

- U-turn: with the row spacing known, tune the turn so the machine lands over the next row (the side sensors / LiDAR then confirm re-entry).

PASS  Acceptance: commanded 10 m matches measured within ~2%, and a U-turn lands within half a row width of centre.

Appendix - Farm Map & Pole Detection Notes

Government farm map

- Export the map to a simple format the planner can read (rows with start/end coordinates). A farm_map.json schema is supported by navigation/global_planner.py.

- Confirm the map's coordinate datum matches the GPS (step 6).

- The planner already supports vacuum-style coverage: start centre -> cover right rows -> return -> cover left rows.

1-metre pole as the row-end marker

- A 1 m vertical pole is detected primarily by LiDAR (a tall, thin vertical cluster) and confirmed by the camera.

- Calibrate the expected pole signature at your turn-trigger distance (step 3).

- This removes the need for printed ArUco/QR/barcode markers at row ends.